Appendix S2 Pseudocode for Neural Network Inference and Stress-Update Procedure

This appendix summarizes the neural-network inference procedure used in constitutive stress integration. Before the loading-path simulation starts, the trained yield-function network and stress-gradient network are loaded together with the corresponding normalization parameters. At each stress-update step, the current principal stresses and history variable are transformed into the network input format, normalized consistently with the training stage, and passed through the trained networks to evaluate the yield function and stress gradient. The elastic trial state is first examined. If the trial state remains admissible, the update is elastic. Otherwise, the procedure dispatches the constitutive correction to either CPA or CPPM.

| Algorithm S2.1: Neural Network Inference and Stress-Update Procedure |  |
| --- | --- |
| 1: | procedure LoadNetworks( |
| 2: | param_file_f, scaler_in_f, scaler_out_f, |
| 3: | param_file_df, scaler_in_df, scaler_out_df) |
| 4: | bundle_f = ReadNetworkParameters(param_file_f) |
| 5: | bundle_df = ReadNetworkParameters(param_file_df) |
| 6: | bundle_f.in_scaler = ReadScaler(scaler_in_f) |
| 7: | bundle_f.out_scaler = ReadScaler(scaler_out_f) |
| 8: | bundle_df.in_scaler = ReadScaler(scaler_in_df) |
| 9: | bundle_df.out_scaler = ReadScaler(scaler_out_df) |
| 10: | return bundle_f, bundle_df |
| 11: | end procedure |
| 12: |  |
| 13: | procedure ForwardInference(x_raw, bundle) |
| 14: | x = Normalize(x_raw, bundle.in_scaler) |
| 15: | for l = 1 to NumberOfLayers(bundle) - 1 |
| 16: | x = DenseTransform(x, bundle.W[l], bundle.B[l]) |
| 17: | if l < NumberOfLayers(bundle) - 1 then |
| 18: | x = Activate(x, bundle.ActFun[l]) |
| 19: | end if |
| 20: | end for |
| 21: | y = Denormalize(x, bundle.out_scaler) |
| 22: | return y |
| 23: | end procedure |
| 24: |  |
| 25: | procedure BuildNetworkInput(sigma_principal, lambda) |
| 26: | sigma_sorted = SortPrincipalStresses(sigma_principal) |
| 27: | p, rho, theta = Convert123ToPrt( |
| 28: | sigma_sorted[1], sigma_sorted[2], sigma_sorted[3]) |
| 29: | x_raw = [p, rho, theta, lambda] |
| 30: | return x_raw |
| 31: | end procedure |
| 32: |  |
| 33: | procedure EvaluateConstitutiveNetworks( |
| 34: | sigma_principal, lambda, bundle_f, bundle_df) |
| 35: | x_raw = BuildNetworkInput(sigma_principal, lambda) |
| 36: | f_val = ForwardInference(x_raw, bundle_f) |
| 37: | grad = ForwardInference(x_raw, bundle_df) |
| 38: | return Scalar(f_val), Vector(grad) |
| 39: | end procedure |
| 40: |  |
| 41: | procedure ComputeElasticTrialState(state, deps, props) |
| 42: | eps_new = state.eps + deps |
| 43: | eps_e_tr = state.eps_e + deps |
| 44: | eps_e_tr_principal, eigvecs = EigenDecompose(eps_e_tr) |
| 45: | sigma_tr_principal = props.Ce_principal * eps_e_tr_principal |
| 46: | sigma_tr = ReconstructTensor(sigma_tr_principal, eigvecs) |
| 47: | return eps_new, eps_e_tr, eps_e_tr_principal, |
| 48: | sigma_tr, sigma_tr_principal, eigvecs |
| 49: | end procedure |
| 50: |  |
| 51: | procedure StressUpdate( |
| 52: | state, deps, props, bundle_f, bundle_df, method, tol, maxiter) |
| 53: | eps_new, eps_e_tr, eps_e_tr_principal, |
| 54: | sigma_tr, sigma_tr_principal, eigvecs = |
| 55: | ComputeElasticTrialState(state, deps, props) |
| 56: | f_trial, grad_trial = |
| 57: | EvaluateConstitutiveNetworks( |
| 58: | sigma_tr_principal, state.lambda, bundle_f, bundle_df) |
| 59: | if f_trial <= 0 then |
| 60: | state.sigma = sigma_tr |
| 61: | state.eps_e = eps_e_tr |
| 62: | state.eps = eps_new |
| 63: | return state |
| 64: | end if |
| 65: | if method == "CPA" then |
| 66: | state = ReturnMapping_CPA( |
| 67: | state, eps_new, eps_e_tr_principal, |
| 68: | sigma_tr_principal, eigvecs, props, |
| 69: | bundle_f, bundle_df, tol, maxiter) |
| 70: | # see Appendix S5 |
| 71: | else if method == "CPPM" then |
| 72: | state = ReturnMapping_CPPM( |
| 73: | state, eps_new, eps_e_tr_principal, |
| 74: | sigma_tr_principal, eigvecs, props, |
| 75: | bundle_f, bundle_df, tol, maxiter) |
| 76: | # second-order term: Appendix S3 |
| 77: | # see Appendix S4 |
| 78: | end if |
| 79: | return state |
| 80: | end procedure |
| 81: |  |
| 82: | procedure RunLoadingPathSimulation( |
| 83: | deps_list, props, bundle_f, bundle_df, method, tol, maxiter) |
| 84: | state = InitializeState() |
| 85: | history = EmptyHistory() |
| 86: | for n = 1 to Length(deps_list) |
| 87: | deps = deps_list[n] |
| 88: | state = StressUpdate( |
| 89: | state, deps, props, |
| 90: | bundle_f, bundle_df, method, tol, maxiter) |
| 91: | SaveState(history, n, state) |
| 92: | end for |
| 93: | return history |
| 94: | end procedure |