Appendix S4 Neural-network-driven CPPM stress-integration algorithm

| Algorithm S4.1: Neural-network-driven CPPM stress-integration algorithm |  |  |
| --- | --- | --- |
| Input: - Previous stress σn, elastic strain εne, plastic strain εnp, equivalent plastic strain λn - Strain increment Δε, elastic stiffness tensor De - Neural networks for yield function fNN(σ,λ) - Gradient neural network fgrad(σ,λ)≈∇σf |  |  |
| Output: - Updated stress σn+1，state variables (εe,εp,λ) |  |  |
| 1. Elastic Predictor |  |  |
| εn+1e, trail=εne+Δε | // Compute trial elastic strain |  |
| σn+1trail=σn+De:εn+1e, trail | // Compute trial stress |  |
| ftr=fNN(σn+1trail,λn) | // Evaluate yield function |  |
| if ftr≤0 : |  |  |
| &σn+1←σn+1trial&εn+1e←εn+1e,trial&εn+1p←εnp&λn+1←λn | // Return elastic solution |  |
| else ftr>0: |  |  |
| 2. Plastic Correction (CPPM) |  |  |
| (εp)0=εnp | // Initialize plastic strain |  |
| Δλ0←0 | // Initialize plastic multiplier |  |
| σ0=Deεn+1-(εp)0 | // Evaluate the elastic trial stress |  |
| Repeat Newton iteration until convergence: |  |  |
| At iteration k: |  |  |
| fk=fNN(σk,λn+Δλk) rk=-εpk+εnp+Δλkak | // Predict the yield-function value using the yield-function neural network |  |
| if ¦fk¦<TOL1 and rk<TOL2 : | // Check yield condition and convergence at k-th iteration |  |
| Δλ←Δλk | // Update the plastic multiplier |  |
| Exit the local k-loop. |  |  |
| else: |  |  |
| ak=fgrad(σk,λn+Δλk) | // Compute gradient of yield function via gradient neural network |  |
| ∂ak∂σ≈a(σk+Δσiei)-akΔσii=1,…,6 | // Compute the second-order derivatives using forward finite differences (Appendix S3) |  |
| Rk=I+ΔλkDe∂ak∂σ-1De δλk=fk-(ak)TRkrk(ak)TRkak | // Compute increment in plasticity parameter |  |
| Δσk=-Rkrk-δλkRkak | // Obtain stress increments |  |
| (εp)k+1=(εp)k-(De)-1Δσk Δλk+1=Δλk+δλk σk+1=σk+Δσk | // Update plastic strain |  |
| k←k+1 |  |  |
| Return to the start of the loop and check whether to terminate the local k-loop. |  |  |
| 3. Update state variables |  |  |
| Δεp=Δλ⋅a | // Compute plastic strain increment |  |
| &εn+1p=εnp+Δεp&εn+1e=εn+1e,trial-Δεp&λn+1=λn+Δλ&σn+1=De:εn+1e | // Update state |  |
| Return: - σn+1,εn+1e,εn+1p,λn+1 |  |  |