Appendix S5 Neural-network-driven CPA stress-integration algorithm

| Algorithm S5.1: Neural-network-driven CPA stress-integration algorithm |  |  |
| --- | --- | --- |
| Input: - Previous stress σn, elastic strain εne, plastic strain εnp, equivalent plastic strain λn - Strain increment Δε, elastic stiffness tensor De - Neural networks for yield function fNN(σ,λ) - Gradient neural network fgrad(σ,λ)≈∇σf |  |  |
| Output: - Updated stress σn+1，state variables (εe,εp,λ) |  |  |
| 1. Elastic Predictor |  |  |
| The elastic stage is identical to that in Appendix S4. |  |  |
| 2. Plastic Correction (CPA) |  |  |
| (εp)0=εnp | // Initialize plastic strain |  |
| Δλ0←0 | // Initialize plastic multiplier |  |
| σ0=Deεn+1-(εp)0 | // Evaluate the elastic trial stress |  |
| Repeat Newton iteration until convergence: |  |  |
| At iteration k: |  |  |
| fk=fNN(σk,λn+Δλk) | // Predict the yield-function value using the yield-function neural network |  |
| if ¦fk¦<TOL1 : | // Check yield condition and convergence at kth iteration |  |
| εn+1p←(εp)k+1 Δεp←Δ(εp)k σn+1←σk+1 Δλ←Δλk | // Updated stress and state variables |  |
| Exit the local k-loop. |  |  |
| else: |  |  |
| ak=fgrad(σk,λn+Δλk) | // Compute gradient of yield function via gradient neural network |  |
| Δλk=fk(ak)TDeak | // Compute increment in plasticity parameter |  |
| Δσk=-ΔλkDeak | // Obtain stress increments |  |
| Δ(εp)k=-(De)-1Δσk (εp)k+1=(εp)k-(De)-1Δσk σk+1=σk+Δσk | // Update plastic strain |  |
| k←k+1 |  |  |
| Return to the start of the loop and check whether to terminate the local k-loop. |  |  |
| 3. Update state variables |  |  |
| εn+1p and σn+1 have already been updated at the end of the local iteration. |  |  |
| &εn+1e=εn+1e,trial-Δεp&λn+1=λn+Δλ | // Update state |  |
| Return: - σn+1,εn+1e,εn+1p,λn+1 |  |  |