Appendix S7 Generation and augmentation of training data for neural network-based yield function modeling

| Algorithm S7.1: Generation and augmentation of training data for neural network-based yield function modeling |  |  |
| --- | --- | --- |
| Input: - Benchmark yield function fb(p,ρ,θ,λ) - Number of level sets Nψ |  |  |
| Output: - Augmented and normalized training dataset Dtrain |  |  |
| 1. Sample yield surface points |  |  |
| Dbase←∅ |  | // Initialize base dataset |
| λk∈{λ1,…,λK} |  | // Define hardening levels |
| for each λk do |  |  |
| for pi in [pmin,pmax] and θj in [-π6,π6] do |  |  |
| Solve for ρij such that: |  |  |
| fb(pi,ρij,θj,λk)=0 |  |  |
| Add (pi,ρij,θj,λk) to Dbase |  |  |
| 2. Perform level-set based data augmentation |  |  |
| Daug←∅ |  | // Initialize augmented dataset |
| for each (p,ρ,θ,λ)∈Dbase do |  |  |
| for each j in [0, 2, …, 2Nψ] do |  |  |
| ζj←jNψ-1 | // ζj = 0 denotes the original yield surface |  |
| &p*←p&ρ*←ρ+ζj⋅ρ&θ*←θ&λ*←λ |  | // Generate new sample |
| f*←ζj⋅ρ |  | // Compute signed distance label |
| ∂f∂σA←∇xf⋅∂(p,ρ,θ)∂σA |  | // Compute stress gradient |
| (p*,ρ*,θ*,λ*,f*,∂f/∂σA)→Daug |  | // Append augmented sample to dataset |
| 3. Normalize input and output |  |  |
| for each feature xi∈{p,ρ,θ,λ,f,∂f/∂σA} : |  |  |
| Compute min(xi),max(xi) |  |  |
| xinorm=xi-min(xi)max(xi)-min(xi) |  | // Apply min-max normalization |
| Store normalization parameters in text files |  | // e.g., f_ins.txt, f_outs.txt, etc. |
| Return: - Final training dataset Dtrain with normalized values and corresponding gradient labels. |  |  |