Supplementary Materials

S1. Implementation Details

To reflect commodity indoor sensing, the robot's proprioceptive channels are perturbed at every control step on top of the LiDAR range noise of Eq. 13. The perturbations are independent uniform random variables with the following envelopes: ±2 cm per axis for position, ±π/50 rad (≈ ±3.6°) for heading, ±1 cm s⁻¹ for linear velocity, and ±π/50 rad s⁻¹ for angular velocity. These magnitudes follow the residual-error specifications of mid-range LiDAR and wheel-encoder odometry, so the policy meets realistic state-estimation drift at every step.

All experiments ran on a single workstation: Windows 11, AMD Ryzen 9 7845HX (12 cores), 32 GB RAM, NVIDIA RTX 4060 Laptop GPU (8 GB VRAM). The software stack is Python 3.9 with PyTorch 2.2.0, PyGame 2.5.2, NumPy 1.24.4, SciPy 1.10.1, and TensorBoard 2.18.0.

S2. Action Space and Parameter Configuration

Table S1: Discrete action set 𝒜: velocity commands and functional role of each action.

| Action | Vl (cm/s) | Va (rad/s) | Functional role |
| --- | --- | --- | --- |
| a(0) | 10 | +2 | In-place left turn |
| a(1) | 50 | +2 | Forward + left turn |
| a(2) | 50 | 0 | Straight cruise |
| a(3) | 50 | −2 | Forward + right turn |
| a(4) | 10 | −2 | In-place right turn |
| a(5) | −50 | 0 | Reverse |
| a(6) | 5 | 0 | Slow approach |

Table S2: Configuration of Core Experimental Parameters.

| Module Category | Parameter | Value | Type | Description |
| --- | --- | --- | --- | --- |
| Network Model | net_width | 64 | Integer | Hidden layer width of the linear layers in the Transformer network. |
|  | T | 10 | Integer | Time window length for dynamic obstacle motion prediction. |
|  | H | 8 | Integer | Number of heads in the multi-head attention mechanism of the Transformer encoder. |
|  | L | 3 | Integer | Number of layers in the Transformer encoder. |
| Simulation Environment | dvc | cuda | String | Device for running the planner (options: cuda / cpu). |
|  | action_type | Discrete | String | Type of action space (options: Discrete / Continuous). |
|  | window_size | 800 | Integer | Pixel dimensions of the simulation environment map. |
|  | D | 800 | Integer | Maximum local planning distance (unit: cm). |
|  | O | 6 | Integer | Number of dynamic obstacles in a single environment instance. |
|  | Obs_R | 14 | Integer | Maximum radius of obstacles (unit: cm). |
|  | Obs_V | 30 | Integer | Maximum velocity of obstacles (unit: cm/s). |
| Obstacle Randomization | ScOV | FALSE | Boolean | Flag to scale the maximum velocity of dynamic obstacles. |
|  | RdOV | TRUE | Boolean | Flag to randomize the motion velocity of dynamic obstacles. |
|  | RdOT | FALSE | Boolean | Flag to randomize the type of dynamic obstacles. |
|  | RdOR | TRUE | Boolean | Flag to randomize the radius of obstacles. |
| Warehouse Task | goal_number | K | Integer | Number of shelf locations to visit before reaching the delivery point (K>1). |
|  | num_episodes | 100 | Integer | Total number of episodes (trials) for the experiment. |
|  | noise | TRUE | Boolean | Flag to add noise to LiDAR observations, simulating real-world sensor noise. |
|  | max_ep_steps | 2000 | Integer | Maximum steps per episode to prevent infinite loops in the planning process. |
| LiDAR Sensor | ld_a_range | 360 | Integer | Maximum scanning angle of the LiDAR (unit: degrees). |
|  | ld_d_range | 300 | Integer | Maximum scanning distance of the LiDAR (unit: cm). |
|  | ld_num | 72 | Integer | Number of laser beams per single scan frame. |
|  | ld_GN | 3 | Integer | Number of beam groups for LiDAR feature extraction. |

S3. SW-DWNS Control Loop

Algorithm S1: SW-DWNS Control Loop (single episode)

Input: Goal sequence 𝒢 = (𝒈1, ..., 𝒈K+1); Pretrained policy πθ; threshold ε; step budget tmax Output: Terminal outcome ∈ {SUCCESS, COLLISION, TIMEOUT} 1: Q ← deque(𝒢); 𝒈 ← head(Q) 2: 𝒔 ← env.reset(𝒈) 3: for t = 1, 2, ..., tmax do 4: at ← πθ(𝒔) 5: 𝒔, collided ← env.step(at) 6: if collided then return COLLISION 7: if ‖env.pos − 𝒈‖2 < ε then ▷ AGS rule (Eq. 15) 8: Q.popleft() 9: if Q is empty then return SUCCESS 10: 𝒈 ← head(Q) 11: return TIMEOUT

S4. Additional Results

Table S3: Average Steps per Successful Episode across Three Layouts (K = 1–5).

| Average Steps per Successful Episode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Layout | Method | K =1 | K =2 | K =3 | K =4 | K =5 |
| Dispersed | A*+DWA | 178.0 | 270.0 | 308.0 | 391.0 | 556.0 |
|  | SW-DWNS | 252.0 | 387.0 | 572.0 | 766.0 | 818.0 |
| Compact-Simple | A*+DWA | 227.0 | 336.0 | 609.0 | 472.0 | 788.0 |
|  | SW-DWNS | 245.0 | 461.0 | 623.0 | 785.0 | 837.0 |
| Compact-Complex | A*+DWA | 178.0 | 348.0 | 433.0 | 563.0 | 657.0 |
|  | SW-DWNS | 335.0 | 515.0 | 664.0 | 819.0 | 897.0 |
| Average | A*+DWA | 420.9 |  |  |  |  |
|  | SW-DWNS | 598.4 |  |  |  |  |

Note. Mean step counts over successful episodes only. ColorDynamic-only is excluded (1.3% full-grid success yields too few episodes for a stable mean).

Figure S1: Step-by-step visualization of one complete successful K = 2 task in each of the three warehouse layouts: (a–d) Dispersed; (e–h) Compact-Simple; (i–l) Compact-Complex. In each group, the four panels show, in order: (a) the initial state at the start point; (b) the state upon reaching the first target; (c) the state upon reaching the second target; and (d) the state upon reaching the delivery station. Green circle: start; red square: delivery station; dash-dot line: robot trajectory; rings: dynamic obstacles; fan: LiDAR scan.